SIMATS ENGINEERING

COURSE NAME: TRANSMISSION LINES AND WAVE GUIDES

COURSE CODE: ECA1505

COURSE OUTCOME COVERED: CO4AT1

NAME : Dhanush B

REG:192412193

Investigate the effect of dielectric loading on the propagation characteristics of a rectangular waveguide

Code

clc;

clear;

close all;

% Rectangular waveguide dimensions

a = 22.86e-3;       % Width in meters

b = 10.16e-3;       % Height in meters

c = 3e8;            % Speed of light

f = linspace(5e9, 15e9, 1000);

% Dielectric constants

er_air = 1;

er_2 = 2.2;

er_4 = 4.0;

% Cutoff frequencies for TE10

fc_air = c/(2*a*sqrt(er_air));

fc_2   = c/(2*a*sqrt(er_2));

fc_4   = c/(2*a*sqrt(er_4));

% Propagation constants

beta_air = zeros(size(f));

beta_2   = zeros(size(f));

beta_4   = zeros(size(f));

% Calculate beta only above cutoff

idx = f > fc_air;

beta_air(idx) = (2*pi*f(idx)/c) .* sqrt(er_air) .* ...

sqrt(1 - (fc_air./f(idx)).^2);

idx = f > fc_2;

beta_2(idx) = (2*pi*f(idx)/c) .* sqrt(er_2) .* ...

sqrt(1 - (fc_2./f(idx)).^2);

idx = f > fc_4;

beta_4(idx) = (2*pi*f(idx)/c) .* sqrt(er_4) .* ...

sqrt(1 - (fc_4./f(idx)).^2);

% Plot propagation constant

figure;

plot(f/1e9, beta_air, 'LineWidth', 2);

hold on;

plot(f/1e9, beta_2, 'LineWidth', 2);

plot(f/1e9, beta_4, 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Propagation Constant \beta (rad/m)');

title('Effect of Dielectric Loading on Propagation Constant');

legend('\epsilon_r = 1', '\epsilon_r = 2.2', '\epsilon_r = 4');

% Display cutoff frequencies

fprintf('Cutoff frequency for air       = %.2f GHz\n', fc_air/1e9);

fprintf('Cutoff frequency for er = 2.2 = %.2f GHz\n', fc_2/1e9);

fprintf('Cutoff frequency for er = 4   = %.2f GHz\n', fc_4/1e9);

OUTPUT: